| Author’s name Ashley Cross Business unit Global Networks & Technology Process owner Tony Johnston | Sub-business unit / Group Wireless Access / Technology Introduction & Lifecycle Issue date 2 July 2026 | Issue number 39 Telstra ID AABAQ-744472851-37 CIP0038 |
| --- | --- | --- |

Summary

The purpose of this document is to outline Telstra’s policy and list Telstra approved components for use in In-Building Coverage (IBC) Distributed Antenna Systems (DAS). Designers must request the latest copy from Telstra before commencing activities on Telstra lead DAS.

This publication has been prepared and written by Telstra Limited (ABN 64 086 174 781), and is copyright. Other than for the purposes of and subject to the conditions prescribed under the Copyright Act, no part of it may in any form or by any means (electronic, mechanical, micro copying, photocopying, recording or otherwise) be reproduced, stored in a retrieval system or transmitted without prior written permission from the document controller. Product or company names are trademarks or registered trademarks of their respective holders. Note for non-Telstra readers: The contents of this publication are subject to change without notice. All efforts have been made to ensure the accuracy of this publication. Notwithstanding, Telstra Limited does not assume responsibility for any errors nor for any consequences arising from any errors in this publication.

Contents

1.	Scope	3

2.	Business Rules	3

3.	Approved Components Requirements	3

3.1.	DAS designed by or on behalf of Telstra	4

3.2.	DAS built by a third party and Telstra is the ‘lead operator’	4

3.1.	DAS is shared with other mobile operators, but Telstra is not the ‘lead operator’	4

4.	Approved Components	5

4.1.	CommScope to Andrew Rebrand	6

4.2.	Radiating Cables	6

4.3.	Lens Antennas	6

4.4.	Coaxial Cables and Connectors	6

4.4.1.	Custom Tails	7

4.5.	Active Components	9

4.5.1.	Telstra Supported ERA CAP-H Configurations	11

4.5.2.	Telstra Supported Mavenir Crossfire A3+E3+N3RU Configurations	15

4.5.3.	Telstra Supported Multi-operator Radio DOT Configuration	17

4.6.	Approved Components Not Covered	18

5.	Appendix	19

5.1.	Telstra Supported ERA Classic CAN Configurations	20

5.1.1.	SISO	20

5.1.2.	MIMO	20

5.2.	(LEGACY, For Reference only) Telstra Supported Mavenir Crossfire HPRU Configurations	21

6.	References	25

7.	Definitions	25

8.	Document control sheet	25

# Scope

Only main components are listed. No consumables or installation materials are included (e.g. hangers, labels, cable trays etc).

# Business Rules

A controlled list of usable components has been developed to ensure that Telstra has the capability to manage and support Distributed Antenna Systems (DAS) where it is connecting. Telstra will not be able to support a DAS that is built with items that are not on this list.

Only the listed equipment in this document can be specified in IBC DAS designs. If a decision is made to allow use of additional equipment this document should be updated to reflect this decision before the equipment is deployed.

This document will be updated from time to time in order to capture changes in

- The IBC DAS technology

- To reflect additional approvals

- Removal of obsolete equipment

- To correct any unintended errors; and

- To supply any missing information when it becomes available

Where there is a need for further clarification or additional approvals please refer to the process owner for review of the issue and any necessary updates to the document.

# Approved Components Requirements

All passive RF components must meet the minimum requirements of the published MCF DAS Design Specifications.

## DAS designed by or on behalf of Telstra

Components must be selected from those listed in Section 4 of this document. Telstra will provide support for systems using these components as they have been tested, approved and have supply agreements or alternative arrangements in place.

Telstra is not able to support DAS that are built with components that are not on this list. Refer to the process owner for review as per Section 2 Business Rules for any additional approvals necessary for designs.

## DAS built by a third party and Telstra is the ‘lead operator’

All passive RF components must meet the minimum requirements of the current published MCF DAS Design Specification and must be approved by Telstra as the ‘lead operator’.

A DAS built with foreign components will require an audit to determine the components that have been used or proposed in that system. The results of that audit will be compared with the DAS approved items list identified in Section 4 of this document.

Any item without an equivalent on the list may not be supportable and may need to be exchanged or have alternative arrangements put in place. If components are being exchanged in a foreign system, then it shall be confirmed that the DAS performance meets MCF DAS Design Specifications after the exchange.

All active DAS hardware must be Telstra approved and be covered by a Product Supply Agreement or have alternative support arrangements in place. Approved active systems are identified in Section 4 of this document.

Refer to the process owner for review as per Section 2 Business Rules for any additional approvals necessary for designs.

## DAS is shared with other mobile operators, but Telstra is not the ‘lead operator’

All passive RF components must meet the minimum requirements of the current published MCF DAS Design Specification and must be approved by the ‘lead operator’ of the DAS.

All active DAS hardware must have been assessed by Telstra for compliance with Telstra technical standards and must be approved by the ‘lead operator’ of the DAS. It is not necessary that the equipment be covered by any Telstra Product Supply Agreements. Approved active systems are listed in Section 4 of this document.

Refer to the process owner for review as per Section 2 Business Rules for any additional approvals necessary for designs.

# Approved Components

The following approved components have been tested for compliance with Telstra’s requirements. They are covered by Product Supply Agreements (PSA) or have an alternative arrangement implemented.

Components are marked with their status for usage in Telstra lead DAS.

| Current | Approved hardware |
| --- | --- |
| Replaced | Approved hardware that may be used, but has been superseded by a preferred component |
| Legacy | Not the current approved hardware but may be left in service. |
| Replace | No longer part of the approved RF design and should be replaced if faulty |
| Special | Approved hardware, by special order with Supplier |

## CommScope to Andrew Rebrand

Parts listed with CommScope as the manufacturer will soon be supplied by Andrew due to the purchase of the Outdoor Wireless Networks & DAS business units by Amphenol. Products listed as CommScope manufacturer in this document with the same part number can be supplied by Andrew. Over time this document will be updated to show Andrew as the manufacturer, with historical allowances for CommScope branded parts.

## Radiating Cables

Radiating cables are available from Telstra’s preferred Suppliers. Cable types are to be selected and designed on a site-by-site basis in consultation with Telstra and the Supplier.

## Lens Antennas

Lens antennas are available from Telstra’s preferred Suppliers. Antenna types and configurations are to be selected and designed on a site-by-site basis in consultation with Telstra and the Supplier.

## Coaxial Cables and Connectors

- Only corrugated coaxial cables should be used for structured cabling of In-Building Systems.

- Extensive testing by Telstra has shown that the following structured cable and connector combinations give the best PIM performance. They are the only combinations approved by Telstra for IBC projects.

- RFS cables with RFS connectors from the approved list (7-16 and 4.3-10)

- RFS Cables with JMA connectors from the approved list (7-16 and 4.3-10)

- Legacy/pre-existing Commscope cables with JMA connectors from the approved list (7-16 and 4.3-10)

- Legacy/ pre-existing Huber and Suhner cables with JMA connectors from the approved list (7-16 and 4.3-10)

- Only approved connectors listed in Section Approved Components may be used.

- JMA CXP type connectors are approved for IBC DAS Indoor Usage Only. This connector type is not suitable for areas at risk of water ingress or dynamic mechanical loading (i.e outdoor areas).

- JMA UXP type connectors are approved for Indoor and Outdoor Usage.

- Please note that N type connectors are no longer supported by Telstra in IBC projects. 4.3-10 connectors are the preferred alternative.

- Where equipment is supplied with N connectors, the connecting jumper cable shall be fitted with an approved N connector (field-fitted JMA or factory fitted no longer than 5m) on one end and 7-16 or 4.3-10 on the other.

- Only factory fitted tail types or alternative lengths of the same variant listed in Section 4 Approved Components may be used.

- Only approved structured cables listed in Section 4 Approved Components may be used in DAS designed by or on behalf of Telstra.

### Custom Tails

Supply of custom tails is available through Rojone Pty Ltd for short lead time and site-specific applications. Specific cable types, connector series, termination type and accessory code are approved by Telstra and may be ordered as per the Rojone Cable Assembly Catalogue in custom lengths.

Figure 1: Rojone Cable Assembly

Approved codes are as per the following table:

| Tail Assembly | Cable Types | Connector Series | Termination Type | Tail Length | Accessory Code |
| --- | --- | --- | --- | --- | --- |
| CAI – Low Intermodulation (IM) tested | 021 – RG223 110 – SCF14-50J 111 – SCF14-50JFN 120 – SCF38-50J 140 – SCF12-50J 141 – SCF12-50JN 240 – LCF12-50J 241 – LCF12-50JFN 036F – FSJ1-50/SF14-50J 036L – LDF1-50 037F – FSJ4-50B/SCF12-50 037L – LDF4-50/LCF12-50J | 18 – SMA 46 – QMA 30 – DIN 7/16 45 – 4.3-10 NEX10 | A – Crimp straight male plug B – Crimp right angle male plug C – Crimp straight female jack D – Crimp right angle female jack | Any length | 08 - Glue Heatshrink for Excellent Strain relief and twist tolerance |

Note 1: RG223, SMA and QMA components only approved for low power interconnect into DAS head ends.

Note 2: Tails for micro radios with NEX10 connectors must use ¼ inch cable.

## Active Components

The listed components have been tested for compliance with Telstra’s technical requirements and are supported by Telstra as lead carrier or when Telstra is not the lead carrier as per the status below.

| Manufacturer | Solution | Description | Status |
| --- | --- | --- | --- |
| Commscope | ION-B |  | Replaced |
| Commscope | ION-M | High power, multi-band, multi-technology, multi-Operator optical RF distribution system supporting 700, 850, 900, 1800, 2100, 2300 and 2600 MHz bands. Tested and approved for Telstra’s 700, 850, 900, 1800, 2100 and 2600 MHz bands only. | Replaced |
| Andrew (formerly Commscope) | ERA | High power, multi-band, multi-technology, multi-Operator optical RF distribution system supporting 700, 850, 900, 1800, 2100, 2300, 2600, 3400-3800MHz bands. Tested and approved for Telstra’s 700, 850, 900, 1800, 2100, 2600, 3400-3800MHz bands only on CAP-H. Only software V5.50 will be supported for Greenfield DAS. Manual channel configuration is required for all bands to work around known issue with ERA’s lack of automatic NR channel scanning and compatibility with source radio MIMO sleep features. | Current |
| Ericsson | Radio Dot System | The Ericsson Radio Dot System (RDS) is a high-performance multi-band, multi-technology, multi-operator DAS solution. Only the following RDS parts are approved for usage: - IRU 8850: Dot 4455 B1 B3 B78K Indoor Connect 8855 to be used along with abovementioned RDS parts to feed donor carrier(s) signal into a lead carrier’s IRU 8850 to enable multi-operator Radio DOT solution: | Current |
| Mavenir | Crossfire | Multi-band, multi-technology, multi-Operator optical RF distribution system supporting 1800, 2100, 3600 MHz bands. Tested and approved for Telstra’s 1800, 2100 and 3600MHz bands on the A3+E3-O+N3RU (integrated antenna variant). Mavenir Crossfire v1.4 software for A3+E3-O+N3RU to be used with Mavenir Crossfire hardware. | Current |
| Mavenir | Crossfire | Tested and approved for Telstra’s 700, 850, 900, 1800, 2100 and 2600 MHz bands only on the HPRU+AU. Mavenir Crossfire v3.2 software for HPRU+AU to be supplied with new Mavenir Crossfire supplied hardware. Greenfields and augmentations to supply through Mavenir. | Legacy |
| Sunwave | Crossfire | Mavenir Crossfire v3.2 software for HPRU+AU to be used for new builds of ex-Sunwave supplied hardware | Legacy |

### Telstra Supported ERA CAP-H Configurations

Telstra will only support the Commscope ERA CAP-H solution with the following conditions:

- Only the CLASSIC CAN, TEN and CAP-H building blocks may be deployed

- Telstra doesn’t currently support WIN and SWITCHING CAN building blocks

- RFD input pre-combining is not supported for greenfield sites

- Maximum of 8 x CAP-H sets per signal source sector under the following conditions

  - Uplink Gain Offset parameter in ERA S/W should be adjusted sufficiently to achieve <5 dB noise rise to Carrier radio interfacing with ERA RFD card

  - UE signal power to Carrier radio > -87 dBm

- Rojone CAI-021<Radio Connector><Radio Termination>46A<Length>-08 to interface 4.3-10 cables to RFD input

- The new WCS-4 DC (7844068-02) and WCS-2 DC (7844067-02) with integrated DCDC converter approved and will operate on SW version 5.50 or higher.

- Single Mode fibre and SFPs only.

- 3400-3800 MHz band is approved.

- For 3400 – 3800 MHz a CSFP+ kit (A7846858) is required from CommScope. This will provide two fibre links from two CAN/TEN OPT ports, to a single SFP on the CAP-H LT/HT. For maximum flexibility ensure 2 fibres per CAP H 35LT/35LT or 35HT/35HT radio is provisioned. Where Telstra is nominated as lead, consult with Telstra PM for head end rack layouts.

- Kerem KT-PDP-A100 must be used

  - Where dual power planes are available, interleave planes between devices Where single plane power is available, a single plane kit is available from Kerem to service more than 8 x devices from PDP.

The basic supported building blocks are summarised in the following figures and detailed in Appendix 5.1. No other combinations are currently supported by Telstra.

CLASSIC CAN

CAP-H Stream A 85/9/18/21

8585/9/18/21

CAP-H Stream B 85/9/18/21

85/9/18/21

CAP-H Stream A 7/23T/26

85/9/18/21

CAP-H Stream B 7/23T/26

85/9/18/21

CAP-H 35LT/35LT Stream A/B

85/9/18/21

CAP-H 35HT/35HT Stream A/B

85/9/18/21

700-2600: Max 8 x CAP-H (SISO or MIMO)
3400-3800: Max 5 x CAP-H (SISO or MIMO)

CLASSIC CAN

Max 2x TEN per Classic CAN (SISO or 2/2)

TEN

TEN

CAP-H Stream A 85/9/18/21

8585/9/18/21

CAP-H Stream B 85/9/18/21

85/9/18/21

CAP-H Stream A 7/23T/26

85/9/18/21

CAP-H Stream B 7/23T/26

85/9/18/21

CAP-H 35LT/35LT Stream A/B

85/9/18/21

CAP-H 35HT/35HT Stream A/B

85/9/18/21

Per TEN:
700-2600: Max 8 x CAP-H (SISO or MIMO)
3400-3800: Max 5 x CAP-H (SISO or MIMO)

CAP-H Stream A 85/9/18/21

85/9/18/21

CAP-H Stream B 85/9/18/21

85/9/18/21

CAP-H Stream A 7/23T/26

85/9/18/21

CAP-H Stream B 7/23T/26

85/9/18/21

CAP-H 35LT/35LT Stream A/B

85/9/18/21

CAP-H 35HT/HT Stream A/B

85/9/18/21

Per TEN:
700-2600: Max 8 x CAP-H (SISO or MIMO)
3400-3800: Max 5 x CAP-H (SISO or MIMO)

(LEGACY, For Reference Only) Telstra Supported Mavenir Crossfire AU+HPRU Configurations

Telstra will only support the Mavenir Crossfire AU+HPRU solution with the following conditions:

- 1 x AU can serve up to 6 x HPRU clusters simulcast from a single Sector using OP 1-6

- Only HPRUs in the same cluster associated with same AU may be daisy chained

- Daisy chaining of HPRU associated with a different cluster or different AU is not supported

- Daisy chaining of Slave AUs and EUs is not supported

- It is recommended to use dedicated duplex/simplex fibre & SFPs from AU (OP1-6) to RU

- Single Mode fibre and SFPs only

- Where Telstra is nominated as lead, consult with Telstra PM for head end rack layouts

- Kerem KT-PDP-A100 must be used

  - Where dual power planes are available, interleave planes between devices

  - Where single plane power is available, a single plane kit is available from Kerem to service more than 8 x devices from PDP

The basic supported building blocks are summarised in the following figures and detailed in Appendix 5.2

HPRU cluster

6 x cluster per AU

AU

HPRU

HPRU

HPRU cluster N

HPRU cluster M

AU

HPRU

HPRU

HPRU

HPRU

HPRU cluster N

AU

HPRU

HPRU

EU/AU

### Telstra Supported Mavenir Crossfire A3+E3+N3RU Configurations

Telstra will only support the Mavenir Crossfire A3+E3-O+N3RU solution with the following conditions:

- A3+E3-O+N3RU solution must comply with CIP0038a01 Mavenir N3RU Design Guidelines which provides additional detail on approved architectures and design rules.

- 1 pair of A3 units can serve up to 90 x N3RU units simulcast from a single sector using OP 1-6

- Up to 6 E3-O units can be connected per A3 pair

- Either a ring or hub & spoke architecture can be used

  - Up to 5 N3RU units can be cascaded in a ring

  - Up to 7 N3RU rings can be connected per E3-O unit in a ring architecture

  - Up to 14 N3RU units can be connected to an E3-O in a hub & spoke architecture

- No direct connection from N3RU to A3 pair

- Daisy chaining of E3-O units is not supported

- Single Mode fibre and SFPs only

- Where Telstra is nominated as lead, consult with Telstra PM for head end rack layouts

- Kerem KT-PDP-A100 must be used

  - Where dual power planes are available, interleave planes between devices

  - Where single plane power is available, a single plane kit is available from Kerem to service more than 8 x devices from PDP

The basic supported building blocks are summarised in the following figures:

### Telstra Supported Multi-operator Radio DOT Configuration

Telstra will only support the Multi-operator Radio DOT solution with the following conditions:

- Solution must comply with CIP0038a04 Multi Operator DOT Design Guidelines which provides additional detail on approved architectures and design rules

- IRU 8850 with DOT 4455 B1 B3 B78K variant is supported for multi-operator solution

- Only lead carrier can digitally interconnect to IRU from the baseband. All donor carriers to connect RF signals to an Indoor Connect 8855 that is managed by the lead carrier of the site

- CIP0038a05 IRU-DOT Mapping Template must be filled at the time of the build

- A IRU 8850 can serve up to 16 DOTs through the CPRI based RDI port

  - Up to 8 DOTs can be DC powered through IRU 8850. If IRU 8850 is used as a power source, RDI port number and DC port number must match

  - Power D608 can source the extra DC power to the DOTs if there are more than 8 DOTs are housed by an IRU 8850

- 25G two fibre working connection between lead carrier baseband to the IRU 8850 and IRU8850 to the DOTs

- Where Telstra is nominated as lead, consult with Telstra PM for head end rack layouts

## Approved Components Not Covered

The listed components have been tested for compliance with Telstra’s technical requirements but are NOT covered by any Telstra Product Supply Agreements, nor alternative arrangements.

These are approved for use in cases where Telstra is not the sponsor of an IBC project, is not the lead carrier and the lead carrier has approved usage of these components.

| Manufacturer | Solution | Description | Status |
| --- | --- | --- | --- |
| BTI (Bravotech) | msBSC-C | Multi-band, multi-technology, multi-Operator optical RF distribution system. Tested and approved for Telstra’s 850, 900, 1800, and 2100 MHz bands only. | Legacy |
| Huawei | SingleDAS | Medium power, multi-band, multi-technology, multi-Operator optical RF distribution system. Tested and approved for Telstra’s 850, 900, 1800 and 2100 bands only. | Replace |
| Zinwave | 3000 | Low power broadband. | Replace |

# Appendix

## Telstra Supported ERA Classic CAN Configurations

### SISO

### MIMO

## (LEGACY, For Reference only) Telstra Supported Mavenir Crossfire HPRU Configurations

Figure 2: Crossfire High Power - 6 Band SISO

Figure 3:Crossfire High Power - 7 Band SISO

Figure 4: Crossfire High Power - 6 Band MIMO

Figure 5: Crossfire High Power - 7 Band MIMO

-

# References

| Document number | Title |
| --- | --- |
| N/A | MCF DAS Design Specifications |
| CIP0038a01 | Mavenir N3RU Design Guidelines |
| CIP0038a02 | N3RU Cascade Power Calculator |
| CIP0038a03 | A3-E3-N3RU Mapping Template |
| CIP0038a04 | Multi Operator DOT Design Guidelines |
| CIP0038a05 | IRU-DOT Mapping Template |

# Definitions

| Term | Definition |
| --- | --- |
| IBC | In-building coverage |
| DAS | Distributed antenna system |
| ICS | Interference cancelling system |
| MCF | Mobile Carriers Forum |
| PIM | Passive intermodulation |
| RF | Radio frequency |

# Document control sheet

Who to reach out to if you have any queries, questions, changes or concerns?

| Name | Tony Johnston |
| --- | --- |
| Position | Senior Network Engineer |
| Email | anthony.johnston@team.telstra.com |

If you have a suggestion for improving this document, please contact the person listed above.

| Issue number | Issue date | Details on the change |
| --- | --- | --- |
| 1 | 09/10/2008 | Initial Release |
| 2 | 12/12/2008 | Add Andrew WXL-5-50 Cable and Connectors |
| 3 | 18/12/2008 | Add Andrew PSA Components (4830000204) |
| 4 | 25/03/2009 | Add Enersus Passive Components |
| 5 | 04/08/2009 | Add RFI Yagi YB815-81 for 3G850 |
| 6 | 10/09/2009 | Add Outdoor Enersus Panels |
| 6a | 13/11/2009 | Add MR8518D |
| 6b | 18/11/2009 | Add Jaybeam 7478000 Log Periodic Antenna |
| 6c | 19/11/2009 | Add Rojone AI-460-D-NF |
| 6d | 24/11/2009 | Add Andrew Node-G938 |
| 6e | 05/01/2010 | Add RFI YB806-94 for GSM |
| 6f | 06/01/2010 | Add Andrew ION-M 85P/18P/21P |
| 6g | 16/08/2010 | Add Argus CNPA303U |
| 6h | 11/03/2011 | Replace the CY076F1V triplexer with the newer version TBC0076F5V |
| 7a | 14/03/2011 | Replace 800-2500MHz Rojone components with LTE Compatible 700-2700MHz components |
| 8a | 23/04/2013 | Update of CommScope (Andrew) components. Add a new section on superseded components so that the history of updates is kept. |
| 9 | 17/07/2013 | Added Andrew DM-30 and DM-60 MIMO panel antennas |
| 10 | 04/06/2014 | Added Section 7 – Approved Components without a PSA Updates roles and responsibilities |
| 11 | 15/07/2015 | Updates to Approved Components List(definitions) |
| 12 | 08/02/2016 | Huawei SingleDAS and Zinwave added to Section 6 |
| 13 | 09/02/2016 | Update to 4.2 noting that Telstra must approve components. Section 7 updated |
| 14 | 05/03/2016 | Update section 4.3, 5 and 6 |
| 15 | 18/04/2017 | Update to approved material list, cables and connectors, deployment application |
| 16 | 19/12/2019 | Updates roles and responsibilities Updated business rules Sunwave Crossfire marked as legacy Added Commscope UW couplers |
| 17 | 09/03/2020 | Clarified approved connector and cable combinations: only connectors on approved list may be used Clarified escalation process for new components: contact author for review if a new component is necessary to deliver project Additions Rojone Load:AMA-5257CL-100W-45A, AMA-5257CL-30W-45A Innovatel Couplers/Splitters: IN-MCPE-MD7038B-10-155, IN-MCPE-MD7038B-13-155, IN-MCPE-MD7038B-15-155, IN-MCPE-MD7038B-20-155, IN-MCPE-MD7038B-30-155, IN-MCPE-MD7038B-5-155, IN-MCPE-MD7038B-6-155, IN-MCPE-MD7038B-7-155, IN-MCPE-MD7038B-8-155, IN-MHSE-MD7038B-155 Innovatel Combiner: IN-HMSE-MD7038B-155 Galtronics Antennas: Extent D5501i, Extent D5778i, Extent D5944i, GI0801-06971, GI0802-06835, Pear M5277i, Pear M5542i, Pear S5379i Pigtail, Pear S5490i, Pear S5606i Pigtail RFS Antennas: I-ATO5-43-698/2700, I-ATO5-43-698/2700M, I-ATP5-43-698/2700, I-ATP5-43-698/2700M |
| 18 | 12/05/2020 | Simplified language of business rules and approved component policy Additions JMA: UXP N type connectors for legacy and non-compliant equipment RFS: Various 7/16 and 4.3-10 multifit and rapidfit connectors; Various jumper cable combinations including 1/4” and 3/8” Kaelus: BSF0020F1V1 band stop filter Ericsson: Radio Dot system ; IRU 2242: RD 4442 B3+B7 or RD 4442 B1+B7; IRU 8846: RD 4479 B78L |
| 19 | 26/05/2020 | Additions Commscope: Various tail combinations JMA: Various tail combinations RFS: Various tail combinations and JFN series cables W&B: Various tail combinations |
| 20 | 01/06/2020 | Removed embedded documents for publication as a PDF Added key describing component usage Added radiating cable requirements |
| 21 | 03/06/2020 | Lens antenna requirements added Added remarks covering supported and tested Telstra bands for active products Marked DDF0035F1V1 as Legacy, BSF0020F1V1 as Current Corrected Supplier for JMA Wireless components JMA Wireless – Teko Systems Group Sirius Teko Telecom approved for third party lead DAS only |
| 22 | 04/06/2020 | Mavenir HPRU Crossfire added, Sunwave Crossfire marked as replace with Mavenir Software |
| 23 | 26/06/2020 | Added: Selected JMA CXP connectors T-2-UW-43-F, T-2-UW-43-M, T-10-UW-43-M-i6, T-30-UW-43-M-i6, T-50-UW-43-M-i6, T-100-UW-43-M-i6 |
| 24 | 07/09/2020 | Changed: Approved list moved to Excel spreadsheet and Locked document from editing Added: Section detailing supported Crosfire HPRU configurations ION-M approved components list Crossfire approved components list Amphenol tails as listed CMAX-DM60-43-UWI53 SJH-100 E14F55P89, E14F20P05 CDS5-43-617/3800, CDS6-43-617/3800, CDS8-43-617/3800, CDS10-43-617/3800, CDS13-43-617/3800, CDS15-43-617/3800, CDS20-43-617/3800, CDS30-43-617/3800 PDS2-43-698/3800-01S, PDS3-43-698/3800-01S, PDS4-43-698/3800-01S |
| 25 | 11/11/2020 | Changed: Corrected description for RFS FRLSZH cable Added: F2A Commscope Tails L5-40-MM CMAX-DM30-43-UWI53 |
| 26 | 24/11/2020 | Changed: Added Telstra S/I to E14F55P89 Marked as legacy I-ATO5-43-698/2700, I-ATO5-43-698/2700M, I-ATP5-43-698/2700, I-ATP5-43-698/2700M Added: I-ATO5-43-698/3800, I-ATO5-43-698/3800M, I-ATO5-43-698/4000, I-ATP5-43-698/3800, I-ATP5-43-698/3800M PDS2-43-617/3800, PDS3-43-617/3800, PDS4-43-617/3800 43M43MS12-0100FFP, 43M43MS12-0200FFP, 43M43MS12-0300FFP, 43M43MS12-0400FFP, 43M43MS12-0500FFP 7M43MS12-0100FFP, 7M43MS12-0300FFP, 7M43MS12-0500FFP |
| 27 | 01/02/2021 | Changed: Clarification of business rules and legacy/existing feeders use Marked as legacy CMAX-D-43-V53, CMAX-OMF1-43-V53, CMAX-DM60-43-V53, QBC0003F1V5, CMAX-3030S-43-V53, CMAX-DMF-43-V53 Added: QBC0017F1V1-1, F1A-HPHMHM-1M, F1A-HPHMHM-2M, F1A-HPHMHM-3M, F1A-HPHMHM-M5 |
| 28 | 24/05/2021 | Added: CMAX-3030S1-43-V53 AFK640-7 0.5M, AFK640-7 1M, AFK640-7 2M, AFK640-7 3M, AFK599-3 0.5M, AFK599-3 1M, AFK1049-2 1M, AFK599-3 2M, AFK599-3 3M Changed: Added various Commscope product Telstra S/I Added Mavenir Crossfre SW version |
| 29 | 13/07/2021 | Changed: Clarified supported versions of Crossfire SW Added: H-4X4-UW-43-AI6, various RFS FFP jumpers, Commscope ERA CAP-H Marked as Legacy: H-4X4-CPUSE-43-AI6, S-2-CPUSE-L-43-I53, S-3-CPUSE-L-43-I53, S-4-CPUSE-L-43-I53, ROJ-074-4-4-716-I Updated RFS parts: Added Telstra S/I Marked as Replaced: Commscope ION-M, various RFS jumpers |
| 30 | 16/08/2021 | Clarified supported PDP for ERA and Crossfire Updated ERA CAN layouts, maximum configurations and supported configurations Updated status of third party managed hybrid DAS |
| 31 | 16/11/2021 | Added E14F05P11, H-4X4-UW-43-Ai6 Added Rojone custom cables section |
| 32 | 11/08/2022 | Added S-2-UW-L-43I53, S-3-UW-L-43I53, S-4-UW-L-43I53, GI1001-B5490 Commscope NEX10 to 4.3-10 jumpers |
| 33 | 21/06/2023 | Corrected RFS FRLSZH tails designator to “Special” Re-added FRLSZH feeder cable Added NEX10 connector to Custom Tails approved list Adjusted SMA Connector Series to 18 (low PIM) from 17 on Rojone advice Added Galtronics GI1002-07234 antenna RFS FFS, Innovatel, H & S, Legend NEX10 to 4.3-10 jumpers Marked as Legacy Galtronics Pear M5277i |
| 34 | 14/12/2023 | Added note that micro radios with NEX10 connectors use ¼ inch cable Added CMAX-DMF4-43-WI53 antenna Approved CommScope ERA 3500MHz RFD & CAP-H configuration CommScope ERA Fibre protective kit. Ericsson IRU 8848 & Dot 4453 Updated IRU 2242 and RD 4422 marked as legacy ERA approved components RFD replaced with new part number CAP-H 7/23/26 F-AC-F1-APE replaced with non-APE variant CAP-H 7/23/26 F-AC-F1-APE replaced with 100MHz 2300 variant (23T) AC power cable and filler panel marked as legacy Braided cable marked as legacy Various part numbers added |
| 35 | 10/05/2024 | Added that ERA channels must be manually configured. Added note that RFD input pre-combining is not supported for greenfield sites. Added Misc sheet to Approved Component Excel Added Galtronics GI604-06837 CommScope CMAX-LHM2-60-i53 CommScope CMAX-LHM2-30-i53 CommScope CMAX-LHM2-3030-i53 CommScope CMAX-LHM2-22-i53 Innovatel 6228G-4F-2DC switch Marked as Legacy CMAX-3030S1-43-V53 CMAX-DM22S-43-V53 CMAX-DM30-43-UWI53 CMAX-DM30-43-V53 CMAX-DM60-43-UWI53 Innovatel 5428G-2 DC switch |
| 36 | 12/09/2024 | Added KA-6035 quad tuneable filter Marked as Legacy All ION-M equipment |
| 37 | 07/11/2024 | Added Mavenir Crossfire A3+E3-O+N3RU Attachment CIP0038a01 Mavenir N3RU Design Guidelines Attachment CIP0038a02 N3RU Cascade Power Calculator Marked as Legacy Mavenir AU+HPRU |
| 38 | 25/02/2025 | Adjusted Section 3 naming to remove duplicate section naming Added section “4.1. CommScope to Andrew Rebrand” Updated ERA SW version 5.50 Updated RFD allocation to accommodate second Telstra n78 carrier Added WCS-4DC new variant with DCDC converter SDX3340Q-DH-43 ¦ E14F06P80 Combiner |
| 39 | 02/07/2026 | Added Section 4 Approved components - Added Radio DOT components - Added Innovatel Tails - Added Andrew SureFlex Tails - Added RFI Passive Components - Added RFS Combiner Section 4.5 IRU 8850, DOT 4455 B1 B3 B78K and Indoor connect 8855 Section 4.5.3 Multi-operator Radio DOT configuration |