# CIP0038a04 Multi-Operator Radio DOT Design Guidelines

| Author’s name Soyeun Jeon Business unit Wireless Engineering Process Owner Ashley Cross | Sub-business unit Wireless Access Issue date 02/07/2026 | Issue number 1 Telstra ID CIP0038a04 |
| --- | --- | --- |

Summary

This document provides guidelines to ensure the multi-operator Ericsson Radio DOT deployments are designed, built, and integrated in a consistent, compliant, and secure manner when Telstra is involved as a lead or a donor operator.

This publication has been prepared and written by Telstra Limited (ABN 64 086 174 781), and is copyright. Other than for the purposes of and subject to the conditions prescribed under the Copyright Act, no part of it may in any form or by any means (electronic, mechanical, micro copying, photocopying, recording or otherwise) be reproduced, stored in a retrieval system or transmitted without prior written permission from the document controller. Product or company names are trademarks or registered trademarks of their respective holders.

Note for non-Telstra readers: The contents of this publication are subject to change without notice. All efforts have been made to ensure the accuracy of this publication. Notwithstanding, Telstra Limited does not assume responsibility for any errors nor for any consequences arising from any errors in this publication.

This publication has been prepared and written by Telstra Limited (ABN 64 086 174 781), and is copyright. Other than for the purposes of and subject to the conditions prescribed under the Copyright Act, no part of it may in any form or by any means (electronic, mechanical, micro copying, photocopying, recording or otherwise) be reproduced, stored in a retrieval system or transmitted without prior written permission from the document controller. Product or company names are trademarks or registered trademarks of their respective holders.

Note for non-Telstra readers: The contents of this publication are subject to change without notice. All efforts have been made to ensure the accuracy of this publication. Notwithstanding, Telstra Limited does not assume responsibility for any errors nor for any consequences arising from any errors in this publication.

1.	Scope	2

2.	Products	3

2.1.	Indoor Radio Unit (IRU)	3

2.2.	Radio DOT	3

2.3.	Indoor Connect 8855	3

2.4.	Power D608	4

2.5.	Hardware Technical specifications	4

2.6.	Hybrid Cable	6

3.	Architecture and design rules	6

3.1.	Overall architecture	6

3.1.1.	IRU Interconnect Architecture	7

3.1.2.	RDS design Responsibilities	7

3.1.3.	MCF Standard Solution	8

3.2.	Rack layout	8

3.2.1.	Example Remote Rack Layout	9

3.2.2.	Hardware Physical Security	11

3.2.3.	RDS Bill of Materials	11

3.3.	Power Supply for RDS Hardware	11

3.4.	Radio DOT cabling	11

3.5.	Ceiling mounted units	11

3.6.	Wall mounting Power D608	14

3.7.	RF Point of Interconnect	14

3.8.	Design rules	15

3.8.1.	Baseband - IRU connection	15

3.8.2.	Donor radio - IC8855 connection	16

3.8.3.	IC8855 - IRU connection	16

3.8.4.	IRU + D608 - DOT connection	16

4.	Integration	17

4.1.	IC8855 Integration	17

4.2.	Baseband integration	17

4.3.	IRU/DOT integration	17

4.4.	Performance Testing	17

5.	Hardware Labels	17

5.1.	Labelling by Multi Operator DAS Constructor for Telstra lead carrier sites	17

5.2.	Labelling by Telstra RBS Constructor	17

5.3.	Cable labelling	18

5.4.	Indoor Connect 8855 Labelling for RBS constructors	18

5.5.	IRU Label for DAS constructors	18

5.6.	DOT antenna label for DAS constructors	19

5.7.	Power D608 Label for DAS constructors	19

5.8.	RF point of interconnect label for RBS constructors	19

5.9.	Label Templates for DAS constructors	19

6.	References	20

7.	Definitions	21

8.	Document Control Sheet	21

# Scope

This document describes guidelines for the Ericsson Multi-operator Radio DOT deployments that will be supported by Telstra as a lead carrier or as a donor carrier. Donor operators may connect through Telstra owned Indoor Connect product to feed donor radio signal into Indoor Radio Unit. Telstra may connect remote radio unit to lead carrier managed Indoor Connect 8855 to join the existing DAS site as a donor carrier.

The intended audience of this document is any entity designing and installing the Radio DOT products built to share with multiple operators. This document is to be read in conjunction with CIP0038 Approved IBC Components and the MCF Design Guidelines for Distributed Antenna Systems. It assumes knowledge of the Ericsson Radio DOT product and DAS design concepts. Relevant Australian Standards and building codes must be followed during the design and installation of DAS products. Any document issues to be raised with the author for resolution.

# Products

This section describes Radio DOT hardware that enables the multi-operator shared Radio DOT system.

## Indoor Radio Unit (IRU)

The IRU is a DC or AC powered device which transports cellular information between the Baseband and the DOT units. It receives donor cellular information in multi-operator scenarios and transmits to shared DOT units. IRU 8850 is deployed and managed by a lead carrier and a single unit can operate up to16 DOTs.

Figure 1 IRU 8850

## Radio DOT

The Radio DOT is a low power radio transceiver which uses the signal and power interface from the IRU over the Radio DOT Interface (RDI). DOT 4455 B1 B3 B78K variant is used for multi-operator DOT deployment to comply with MCF Standard – 1800, 2100, 3600MHz 2Tx 2Rx solution and managed by a lead operator. DOT 4455 has electrical and optical data ports which cannot be used for data traffic simultaneously, but both ports can be used at the same time if the electrical port is used for power only.

Figure 2 Radio DOT 4455 B1 B3 B78K

## Indoor Connect 8855

The Indoor Connect 8855 enables donor operators to plug in Ericsson or other suppliers’ RF signal sources to support the multi-operator Radio DOT System (RDS). The Indoor Connect 8855 supports up to four frequency bands and RAN signals can be broadcasted to up to four IRUs through the CPRI interface on the Indoor Connect 8855.

Figure 3 Indoor Connect 8855

The lead carrier is responsible for configuration and fault management of IC8855 on site.

When connecting a remote radio unit to the IC8855 as a donor operator of the site, input power from the donor radio must be less than 30dBm per port and the attenuation between the radio and IC8855 must not exceed 20dB. Refer to 3.8.2 for more details on RF conditions.

## Power D608

IRU 8850 has 8 available DC ports to power radio DOTs. When a DOT is connected to the IRU DC port, the DC port number must match with the RDI port that the DOT is connected to. If there are more than 8 DOTs from a single IRU 8850, a power D608 can be used to supply eight additional Power over Ethernet (PoE) ports. D608 converts AC or -48V DC to 56V DC to power radio DOTs with the total PoE output power of 799.2W (99.9W for each PoE++ port).

Figure 4 Power D608

## Hardware Technical specifications

Radio DOT hardware maximum power consumptions, power supply requirements, and dimensions are as below:

Table 1 Power supply specification of the RDS

| Connector | Hardware | Description | Recommended CB |
| --- | --- | --- | --- |
| AC Power | Indoor Connect 8855 | 100-240 V AC, Max 2A, 50-60Hz | - |  |
|  | IRU 8850 | 100-240 V AC, Max 10A, 45-65Hz | - |  |
|  | D608 | 100-240V AC, Max 10A | - |  |
| DC Power | Indoor Connect 8855 | -48V DC, Max 3A | 10A |  |
|  | IRU 8850 | -48V DC, Max 36A | 25A |  |
|  | D608 | -48V DC, Max 18.5A | 25A |  |

Table 2 Dimensions and Weight for the RDS

| Unit | Mechanical detail |  |
| --- | --- | --- |
| Indoor Connect 8855 | 66 x 441 x 364 mm (H x W x D), Weight 7900g |
| IRU 8850 | 65 x 442 x 396 mm (H x W x D), Weight 10900g |
| Power D608 | 43.5 x 448 x 290 mm (H x W x D), Weight 5600g |
| Radio DOT 4455 B1 B3 B78K | 230 mm Diameter, Weight 2300g |

Table 3 Hardware Power Consumption

| Hardware | Max Power Consumption: -48V DC | Max Power consumption: AC | Max Heat Dissipation |
| --- | --- | --- | --- |
| Indoor Connect 8855 | 100W (Max) |  | 108W (Max) |
| IRU 8850 | 1200W (Max with all 16 DOTs connected) | 1330W (Max with all 16 DOTs connected) | 456W (Max) |
| DOT 4455 B78K B1 B3 | 75W (Max) |  | 71W (Max) |  |

Table 4 SFP specification

| - Type | - Product Number | - Fibre Detail | - Wavelength | - Max Supported Distance | - Launched Power | - Link Budget | - Average Receive Power Range |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - 25GBASE-LR | - RDH10275/3 | - Type: SMF - Count: 2 - Connector: LC | - Min: 1295nm - Max: 1325nm | - 10km | - -5.0dBm (Min) - +2.0dBm (Max) | - 6.3dB | - Min: -12.5dBm |

## Hybrid Cable

Hybrid cable is used to connect from the IRU to a Radio DOT providing power and the radio DOT interface signal. Cable size is limited to 12-18AWG due to the connector size. Telstra recommends 16AWG for easy cable management. 12AWG is recommended only for the trunking to get the extra distance if required. Pre-terminated optical cable may be used to ensure the cable build quality and reduce splicing costs.

Table 5 Hybrid Cable details

| Cable wire size (mm²/ AWG) | Max Distance allowed (m) | Cable current tolerance |  |
| --- | --- | --- | --- |
| 1 mm² / 18 AWG | 160 m | PoE end 50 – 57V DOT end 42.5 – 57V |
| 1.5 mm² / 16 AWG | 255 m |  |
| 2.5 mm² / 14 AWG | 465 m |  |
| 4 mm² / 12 AWG | 750 m |  |

# Architecture and design rules

## Overall architecture

Multi-operator RDS architecture comprises of Indoor Connect, IRU and radio DOTs. Indoor Connects shall be in the central communications room within a secure cabinet. The lead carrier will plug baseband CPRI directly into IRU and donor operators will feed the RF through Indoor Connect. RF point of interconnect is installed outside of the secure cabinet for donor radios to be connected. IRUs and DOTs shall be placed in the distributed area with fibre hub connected to the fibre patch in the central communication room.

Figure 5 Multi-operator RDS high level architecture

### IRU Interconnect Architecture

The current approved IRU interconnect architectures are detailed in the below table.

Table 6 IRU Interconnect Architectures

| Operator type | Digital interconnect | RF |
| --- | --- | --- |
| Telstra – Lead Carrier | √ |  |  |
| Telstra – Sharing Carrier |  | √ |  |
| Optus – Sharing Carrier |  | √ |  |
| TPG – Sharing Carrier |  | √ |  |

### RDS design Responsibilities

Table 7 Design Responsibilities for RBS plug-in constructors and DAS constructors

| Design Status | RDS Component | RBS plug in – Lead/Donor carriers | 3rd Party DAS builder |
| --- | --- | --- | --- |
| Installation | RBS Equipment | √ - Lead carrier |  |  |
|  | IRU |  | √ |  |
|  | D608 |  | √ |  |
|  | DOT |  | √ |  |
|  | IC8855 and RF POI | √ - Lead carrier |  |  |
|  | Source radios/RF conditioning elements | √ - Donor carrier |  |  |
| Cabling/Connection | RBS – IRU | √ - Lead carrier |  |  |
|  | IRU/D608 – DOT |  | √ |  |
|  | IC8855 – IRU | √ - Lead carrier |  |  |
|  | IC8855 – RF POI | √ - Lead carrier |  |  |
|  | Source radio – RF POI | √ - Donor carrier |  |  |
| Labelling | RBS Equipment | √ - Lead carrier |  |  |
|  | IRU | √ - to be replaced by Lead carrier | √ - Labels as per DAS builder template |  |
|  | D608 | √ - to be replaced by Lead carrier | √ - Labels as per DAS builder template |  |
|  | DOT |  | √ - Labels as per DAS builder template |  |
|  | IC8855 and RF POI | √ - Lead carrier |  |  |

### MCF Standard Solution

- Radio DOT solution can serve the MCF 2x2 MIMO standard solutions option 2. This option allows 1800, 2100, and 3500MHz band to be radiated in sites with low to medium capacity demands.

Figure 6 MCF 2025 Standard Solution option 2 - 2Tx 2Rx MIMO

## Rack layout

Rack layout to be customized per site to accommodate the site requirement such as the number of DOTs required per sector, rack spacing, power source distribution, etc.

General Notes

- IRUs are recommended to be installed in distributed locations throughout a building for flexible, scalable deployment and shorter hybrid cable run

- IC8855s to be installed in a centralised location for ease of connecivity to management port and to donor radios

- RF point of interconnect to be installed at the top of the rack in the centralised location along with RBS equipment. IC8855 to be connected to the back of the RF point of interconnect at the time of commissioning by lead carrier so that the donor radios can be plugged in without opening the secure cage

- RG223 RF cable to be used to connect between IC8855 (Right angled, QMA) and RF PoI (Straight through, 4.3-10). Refer to CIP0038 document for details on the approved cable list

Installation Notes

- The number of IC8855 to be installed in a security cage may vary in each site depending on the site configuration and operator agreement.

- Specific fibre management tray to be used: WBT P/N: TC247007-40 (Telstra Internal Use Only: Serial / Item 40012102)

- Cable management tray to be used for extra hybrid cable loops as required: WBT P/N: TC3093M (Telstra Internal Use Only: Serial / Item 40001399)

### Example Remote Rack Layout

Below is an example of distributed remote rack layout to house IRU and D608. Where Telstra is nominated as a lead carrier, consult with Telstra PM for any concerns with the rack layout.

Figure 7 Example rack layout for IRU8850 and D608

- IRU to be installed in a remote rack close to the DOT installation. Power D608 to be installed along with IRU as required.

- Remote rack to be installed in well-ventilated room to avoid heat trap

- Install cable guide with both IRU and D608 for easy management of the hybrid cable. If no D608 is required, both CPRI and DC cable to be wrapped on single cable guide. If D608 is in use, DC cable to be guided by D608 and CPRI cable to be guided by IRU cable guide

- When installing hybrid cable, leave at least 1 EXTRA LOOP should be kept either on top of the rack, riser, or roof area depending on the space available for future flexibility. Store the excessive cable close to the DOT install location or consider between IRU space, under or behind remote rack where space available

### Hardware Physical Security

RDS hardware providing network connectivity to Telstra internal network must be physically secured to prevent tampering of hardware or data.

Distributed remote IRU

- Remote racks are recommended to be equipped with the security door

- Only authorized personnel can access the equipment installed in the rack cage

- Consider installing a master key cabinet in a secure location to store remote rack keys. Keys to be handed to the lead carrier.

Central comms room rack

- When Telstra is a lead carrier, IC8855 to be deployed in trusted infrastructure e.g. IBC security cage

Radio DOT

- Radio DOTs are installed at height where public cannot easily reach

### RDS Bill of Materials

Each site will be provided with unique BoM list by the RF engineer to suit the overall design of the site. Refer to the attached example BOM for RDS hardware. Scale the BOM quantity as required.

To initiate the engagement with Telstra Enterprise for purchasing, design enquiries or any other concerns, contact Telstra LRM Enquiries mailing list - LRM.Enquiries@team.telstra.com

## Power Supply for RDS Hardware

For IRU8850, IC8855, and Power D608, both DC and AC power ports are located on the lefthand side of the hardware and can be powered by either depending on the site requirements. Port DC power uses -48V 3 wire-ET type cable. Port AC power requires male C14 port type cable. Choose the power option depends on the rack and the install location requirements. Don’t mix AC and DC power in the same rack.

DOT4455 uses PoE and can be connected to the DC port on an IRU8850 or D608 using a RJ45 DC connector. There are 8 DC port available on an IRU8850 therefore if more than 8 DOTs are installed off an IRU, a separate power source is required such as a D608.

## Radio DOT cabling

All cables in the Radio DOT interface (RDI) channel must fulfill the following:

- All cables must follow local fire safety requirements

- All RDI cables are handled as standard TP cables in case of separation to other cables, as defined in ISO/IEC 14763-2

- RDI cables are installed separated from sources of electrical disturbance

  - >100mm from electrical cables, such as mains power cables and non-power-limited fire alarm circuits

  - >130mm from lamps

  - >1000mm from frequency induction heating

  - Do not install RDI cables near high frequency light bulbs or ballasts

RDI connection between IRU and the DOT using a 2-fibre working 25G link SFP.

- The following are general requirements for the hybrid cable:

  - RoHS (2002/95/EC) compliant

  - Compliant to GR20 Generic Requirements for optical fibre and optical fibre cable

  - Marked for easy recognition

- Cable bending radius must be applicable for installation in the Radio DOT. The bending radius is typically 3 times the cable diameter. Verify the bending radius with the cable datasheet.

## Ceiling mounted units

Minimum mounting distances are required to minimize radio disturbance and to obtain optimal operational performance.

Figure 8 Minimum distance to other Radio DOTs

Figure 22 describes minimum distance between DOTs of different frequency bands to minimize radio disturbances. As per the recommendations, multi-operator compatible DOT 4455 should have minimum distance of 10 cm.

Figure 9 Minimum Distance to other System Antennas

Figure 23 describes minimum distance required between the DOT and other RF device to minimize radio disturbances.

Figure 10 Minimum Distance to Walls & Obstructions

Figure 24 describes minimum mounting distances to walls and other obstructing objects such as a wall to minimize interfering reflections.

Figure 11 Minimum distance to Ceilings

When mounting the DOT on a wall, the top of the bracket must be at a distance (A) of at least 20 cm from the ceiling as shown in Figure 25.

Figure 12 Bracket for hard surfaces

The standard bracket described in Figure 26 is used for installation of DOT4455 on hard surfaces such as concrete ceilings, on pipes, rails, and T-Bars. The installation of the DOT on a metal rod requires an additional bracket shown below in Figure 27.

Figure 13 Metal rod bracket

## Wall mounting Power D608

Power D608 can be wall mounted using a wall mounting bracket if limited space is available to install a remote rack or D608 is installed away from IRU. When wall mounting the unit, allow minimum of 15cm of clearance in the back to allow for good airflow from the fan tray.

Power D608 is to be installed indoor only, where not easily accessible by the public such as near a cable riser or communications cabinet.

Take off the two rack brackets already installed on the Power D608, then install the wall bracket in the centre of the side of the unit.

Figure 14 Wall mounted D608

## RF Point of Interconnect

RF point of interconnect (PoI) shall be installed at the top of the RBS rack for donor operators to connect the radio signal without accessing IC8855 through the IBC secure cage. Connection between IC8855 and RF PoI to be pre-patched at the time of site commissioning. Due to the space restrictions between the side rail of the rack and secure cage, only RG223 cable is recommended to be used.

Expect additional loss up to 2.11db @ 3.6GHz introduced from the patch panel plus 2m patch cable to IC8855.

Figure 15 Example RF Point of Interconnect

Panel should be labelled clearly so the donor operators can connect the radio without any confusion.

## Design rules

- Telstra requires the following design rules to be followed to ensure a suitably functioning RDS solution. Consult with Telstra PM for any site-specific requirements.

- IRU 8850 spacing requirements as follows:

  - The IRU must have minimum 100mm of clearance in the back for good airflow from the fan tray

- Indoor Connect 8855 spacing requirements as follows:

The minimum spacing between the Indoor Connect 8855 and the rear rack wall is 150mm

- Power D608 spacing requirements as follows:

  - The minimum extra space for cable routing is 100mm for cables from the front of the unit

  - The recommended space for easy installation and inspection is to have more than 350mm in front of the unit and 150mm behind the unit

- End to end distance limitation as below:

  - End to end maximum distance cannot exceed 5km

Figure 16 End to End RDS connection

Closest DOT to Farthest DOT distance cannot exceed 2.5km

The mapping template (CIP0038a05) is to be filled by DAS constructors and RBS constructors to provide an end-to-end connectivity of the RDS site

### Baseband - IRU connection

- Each operator manages its own cell configuration. Only one baseband connection to a shared IRU is supported

- Cascaded IRU is not supported

- 25G link is required for the connection between IRU to Baseband.

- IRU data port allocations for participating operators as below:

  - Data 1 - Lead carrier baseband

  - Data 2 - Lead reserved

  - Data 3 - Donor #1 IC8855

  - Data 4 - Donor #2 IC8855

### Donor radio - IC8855 connection

Telstra managed Indoor Connect 8855 to be installed inside a secure cabinet in a designated telecommunications room

- Duplex port to be used on each RF input port pair

- RF port allocation for the spectrum input as below:

  - Port 1A/1B duplex – 1800MHz

  - Port 2A/2B duplex – 2100MHz

  - Port 3A/3B duplex – 3600MHz #1

  - Port 4A/4B duplex – 3600MHz #2

- RF condition for the connection between RRU to IC8855 must follow below table

### IC8855 - IRU connection

- 10G link must be used for the connection between IC8855 to IRU

- IC8855 is operating in a broadcast mode where it can be connected up to four IRUs at most serving the same sector. Scale IC8855 as required.

### IRU + D608 - DOT connection

Data connection must only be by fibre, no CAT6. The RJ45 port is used to power the DOT only

IRU 8850 can power up to 8 DOTs, and an additional power source is required, e.g. D608 or alternative, if more than 8 DOTs are fed through a single IRU

- IRU 8850 and D608 can be either AC or DC powered, however AC and DC power cannot be mixed in the same rack

  - Consider building owner’s power backup options when deciding the power source

IRU 8850 and D608 can be installed in a 19-inch rack in the comms room or a wall mounted cabinet

The DOT 4455 B1B3B78 variant is approved for MCF solution

- Both Fibre and DC cable are required for the connection between IRU and DOT

  - Single mode optic fibre with LC connectors to be used for radio data transmission. Fibre can be pre-terminated or terminated on-site with LC connector toolkit.

  - Two copper wires to be terminated with RJ-45 connector on both IRU and DOT end.

Leave at least one extra loop on each end of the cable when terminating the hybrid cable for future adjustment. The loop can be left on top of the rack, on the roof area, or on the cable management tray, depending on the availability of the site

If hybrid cable is being terminated near IRU where D608 is installed along with the IRU, D608 must be located in the next available rack unit above the IRU

The IRU 8850 and D608 should always be installed with a cable guide. If no D608 is in use, both DC and Optic cables are to be guided by the cable guide in front of the IRU 8850. If a D608 is in use, bundle DC cable on the D608 cable guide and Optic cable on the IRU 8850 cable guide

- Connect cables (RJ45 and/or SFP+ connection) through the bracket before inserting the DOT in the bracket for easy cable management during installation

A 25G link is required for the connection between DOT to IRU and IRU to Baseband.

When RDI port 1-8 is used to connect DOTs and IRU PoE port is used to power the DOTs, the RDI port used and PoE port used must match

When RDI port 9-16 is used along with D608, the DC cable must be connected to the D608 sequentially

- The DOTs must be installed in ceiling panels, on hard ceiling such as concrete surfaces, on pipes, and rails where the installation location can support the DOT unit weight plus the weight of the mounting brackets

- Avoid installing the DOT in places with unventilated lofts, with heat traps, or where the product is exposed to direct sunlight through windows

- Do not install DOTs with metal objects nearer than 1m in front or 0.2m beside them to avoid passive intermodulation interference (PIM). PIM can also be generated through partial contact of metal

- Do not install DOTs near high frequency light bulbs or ballasts

- Keep the DOT a minimum of 0.5m away from items such as fire sprinklers and metal chains

# Integration

When Telstra is the lead operator, RDS units including IRU, Radio DOTs and IC8855 are configured remotely through the connected baseband unit during the integration process.

When Telstra is a donor operator, DAS hardware integration will be implemented by the lead operator. Telstra will feed RF input to lead operator’s point of interconnect to join the DAS.

## IC8855 Integration

IC8855 is configured and managed by the lead operator of the site. Participating operators need to work together to align the spectrum and power gain configuration.

## Baseband integration

- No special configuration is required for multi-operator RDS baseband integration since each operator cells are managed individually. Multi-operator RDS supports standard 2x2 MIMO deployment as per MCF Design Guidelines for Distributed Antenna Systems.

## IRU/DOT integration

The lead carrier does not require any special configuration on IRU or DOT to enable multi operator capability. IRU and DOT configuration is managed by lead carrier’s baseband therefore IRU and DOT connectivity can only be validated once the baseband is integrated.

## Performance Testing

Call tests are to be performed once the lead carrier base station is integrated to confirm the site build. The lead carrier’s end-to-end connectivity between the IRU and DOT is to be tested before testing the donor operator’s signal through the Indoor Connect 8855 to ensure the build of the shared infrastructure is correct.

# Hardware Labels

## Labelling by Multi Operator DAS Constructor for Telstra lead carrier sites

- Wrap-around labels to be used when labelling any cables

- Every cable connection (e.g. fibre, power, ethernet etc.) shall be labelled with an origin and destination location at both ends of the cable

- Cables connected to IRU and DOT shall be labelled with origin and destination at both ends of the cable

- Hybrid cables to be individually labelled for fibre connection and power connection with origin and destination at both ends of the cable

- For identification of RDS units, attach labels on static elements (for example on an adjacent wall or with label holders mounted under rack mounting screws)

All DOTs shall be labelled with the assigned label from iBwave and the connected IRU number and the RDI port number. The label must be attached in a viewable location to ensure easy identification for maintenance purposes

- Example labels can be found in section  5.9 Label Templates for DAS constructors

## Labelling by Telstra RBS Constructor

RBS and Rack labels to be attached as per Telstra’s labelling standard

- Wrap-around labels to be used when labelling any cables

- Cables connected to fibre patches shall be labelled with origin and destination at both ends of the cable

- IRU and power D608 labels labelled by DAS constructors to be replaced with the label as per Telstra Radio DOT installation MOP with site information

- For identification of IRU units and Indoor Connect 8855, attach labels on static elements (for example on an adjacent wall or with label holders mounted under rack mounting screws)

## Cable labelling

- Wrap-around labels to be used when labelling any cables

- Every cable connection (e.g. Fibre, power, ethernet, etc) shall be labelled with an origin and destination location at both ends of the cable

- Cables between RBS equipment to IRU, and IRU to IC8855 shall be labelled by RBS constructors as per Telstra Radio DOT installation MOP with origin and destination location at both ends of the cable

- Fibre and DC cables in hybrid cables between IRU and DOT shall be labelled individually with origin and destination location at both ends of the cables by DAS constructors

- Example labels can be found in section  5.9 Label Templates for DAS constructors

## Indoor Connect 8855 Labelling for RBS constructors

- IC8855 shall be labelled with the donor operator information as per Telstra Radio DOT installation MOP

- IC8855 shall be labelled with the destination IRU/IRUs

## IRU Label for DAS constructors

- IRU shall be labelled at the time of hardware installation by DAS constructor as per below example label. Telstra RBS constructor to replace the label to include RBS information as per Telstra standard later at the time of plug in construction

Example of the shared IRU (Telstra lead, Optus and TPG as donor) label for DAS constructors is depicted below. Telstra RBS constructor to replace the label as per Telstra Radio DOT MOP.

A = Lead Operator code, TLS for Telstra

B = The IRU8850 number in DOT build

C = Donor Operator Code to the IRU8850, TLS, OPT, TPG, connected to IRU data port 3 or data port 4

D = The IC8855 number in DOT build

FL = Floor Number

SS = Suite Number

RR = Rack Number

SR = Subrack Number

## DOT antenna label for DAS constructors

DOT shall be labelled with the iBwave allocated name for each antenna and IRU connection details by DAS constructors. The label details to be recorded in the IRU-DOT mapping template in 3.8 Design rules

Examples of the DOT label is depicted below:

{iBwave name}-RD{IRU Number}-{IRU port number}

Eg. If the DOT is located on level 23 according to iBwave, DOT named A7, connected to IRU number 2 RDI port 8;

## Power D608 Label for DAS constructors

Example of the D608 label is depicted below for DAS constructors. Telstra RBS constructor to replace the label as per Telstra Radio DOT MOP.

A = Lead Operator code, TLS for Telstra

B = The IRU8850 number in DOT build that D608 serving DOTs are connected to

N = The D608 number in DOT build

FL = Floor Number

SS = Suite Number

RR = Rack Number

SR = Subrack Number

## RF point of interconnect label for RBS constructors

The RF point of interconnect label is for the donor operator to identify the correct ICI8855 for the radio connection. In addition to the panel label, each RF port on the point of interconnect shall be clearly labelled next to the port with a port number corresponding to the port label of the connected IC8855.

- RF point of interconnect shall be labelled with the donor operator information as per Telstra Radio DOT installation MOP.

## Label Templates for DAS constructors

# References

| Document number | Title |
| --- | --- |
|  | MCF Design Guidelines for Distributed Antenna Systems |
| CIP0038 | Approved IBC Components |
| CIP0038a05 | IRU-DOT Mapping Template |

# Definitions

| Term | Definition |
| --- | --- |
| RDS | Radio DOT System |
| RDI | Radio DOT Interface |
| IRU | Indoor Radio Unit |
| IC8855 | Indoor Connect 8855 |
| MCF | Mobiles Carrier Forum |
| RBS | Radio Base Station |
| RRU | Remote Radio Unit |  |
| PIM | Passive Intermodulation |  |
| RF | Radio Frequency |  |

# Document Control Sheet

If you have a suggestion for improving this document, please contact the person listed below.

| Name | Soyeun Jeon |
| --- | --- |
| Position | Network Engineer |
| Email | Soyeun.jeon@team.telstra.com |

| Issue Number | Issue Date | Details on the change |
| --- | --- | --- |
| 1 | 02/07/2026 | Initial release |